Wine Maker

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

11-3051.00

RAPIDS CODE

0034

| Work Process Schedule |  |  |
| --- | --- | --- |
| Perform winery cleaning and sanitation Calculate dilutions to prepare cleaning solutions. Perform cleaning and sanitation of winery equipment, including pumps, lines, processing equipment, filters, bottling equipment. |  |  |
| Assess winery hazards and maintain safety procedures Operate machines to process materials in compliance with applicable safety, energy, or environmental regulations. |  |  |
| Operate grape processing equipment Set up and operate sorting table, destemmer-crusher, elevator, must pump and press to process grapes. |  |  |
| Collect samples of materials or products for testing Collect representative samples for the purpose of analysis. Perform vineyard sampling and grape analysis. Monitor Alcoholic Fermentation and Malolactic Conversion conditions and duration. Perform finished wine chemistry panels to ensure quality control. Conduct bench trials of winemaking additions, blends, and other treatments. |  |  |
| Operate grape and wine handling equipment Setup and operate pumps and lines to transfer grapes, must, and wine to bins, barrels, and tanks. Set up and secure tanks to receive grapes must or wine. Set up or adjust glycol chiller to regulate tank temperatures. Prepare new and used barrels for wine storage. Perform winemaking tasks such as cap management, barrel stirring, tank agitation, and barrel topping. Perform sparkling wine production tasks such as riddling, disgorging, corking, and caging. |  |  |
| Operate wine bottling equipment Setup and operate bottle fillers, corkers, capsulers, and labelers. Load cases onto pallets. |  |  |
| Calculate, measure, and perform winemaking additions Perform yeast, bacteria, and nutrient additions. Amend grape must using sugar, water, and tartaric acid. Perform fining agent and wine stability additions. |  |  |
| Inspect equipment and perform repairs Inspect machines or equipment for hazards, operating efficiency, malfunctions, wear, or leaks. Install, maintain, or repair hoses, pumps, filters, or screens to maintain processing equipment, using hand tools. Assemble fittings, valves, bowls, plates, disks, impeller shafts, or other parts to prepare equipment for operation. |  |  |
| Record operational or production data Maintain logs of winery operations, analysis results, and addition trials, test results, or shift production for entry in computer databases. |  |  |
| Materials handling and control Safely use pallet jacks, ladders, hand trucks, cat walks, overhead doors. Store, move, secure, and use compressed gas cylinders and compressed air. |  |  |
| Perform vine and vineyard management tasks Perform vine training and management tasks such as pruning, shoot thinning, shoot positioning, cluster thinning, canopy management, and harvesting. Manage vineyard weeds using cultural and chemical approaches. |  |  |
| Provide wine tastings and service to wine consumers Conduct wine tastings and provide customer service. Use point-of-sales system to conduct sales transactions. |  |  |
| Totals |  | OJT HRS 0 |

Wine Maker

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

11-3051.00

RAPIDS CODE

0034

| Related Instruction Online | RSI HRS |
| --- | --- |
| Winemaking and Fermentation | 45 |
| Wines of the World I | 15 |
| Quantitative Reasoning | 60 |
| Wines of the World II | 15 |
| Plant Biology | 60 |
| Establishing a Vineyard | 60 |
| Grapevine Pruning | 15 |
| Wines of the World III | 15 |
| Maintaining a Vineyard | 45 |
| Vineyard Diseases & Diseases | 45 |
| Winery Business Operations | 45 |
| Chemistry | 90 |
| Wine Chemistry | 45 |
| Methods of Wine Analysis | 30 |
| Totals | RSI HRS 590 |