LINE ELECTRICIAN

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

49-9051.00

RAPIDS CODE

WSATC-1807

| Work Process Schedule |  | OJT HRS |
| --- | --- | --- |
| Poles, Arms, and guys Dig pole holes in ordinary soil, rock, hard pan, swamp Locate poles with proper regard to property lines, sidewalks, ditches, etc. Load and unload poles on trailers, trucks, or cars Roof and frame poles Set poles with derrick and with pikes Face, straighten and line in poles Place 'heel and toe' on self-supporting poles Fill and tamp around pole Stub a decayed pole Move a pole by trenching Test poles for decay, using test bar Replace poles by digging new hole beside old pole Replace poles by pulling old butt and reaming hole Select proper arm for job Install single arm and buck arm Install double arm with space bolts Install alley arm Install pins and deadend bolts in arm Make up guy wire body and tails using clamps and also by serving wire Install all types of anchors-legs, expanding cone, swamp, etc. Install all types of guy-sidewalk, span, arm, anchor, etc. Methods of proper rigging. |  | 1.6K |
| Conductors, switches, protective devices Stringing of conductors Bare copper and aluminum, W.P. copper and aluminum Splicing conductors, copper and aluminum Dying in conductors, copper and aluminum Making conductor to conductor taps, copper and aluminum and combinations of both Install common types of insulators Trimming trees to clear wires Remove conductors being replaced Install single pole disconnecting switches Install gang-operated switches Install line fuses Install pole-mounted oil switches Install capacitor banks Install lightning arrestors Install and operate line reclosers Install and operate line sectionalizing apparatus |  | 1.7K |
| Transformers Install cutouts and proper fuses Install and connect a single-phase transformer for 120-volt two-wire load and 120/240-volt three-wire load Install and connect an additional transformer to secondary bus fed by other transformers Install and connect three transformers-wye-delta Install and connect two transformers-wye-delta Connect transformer bank to supply 240-volt three-phase and 120/240-volt single-phase service Connect transformer bank to supply combination 208-volt three-phase and 120/208-volt single-phase service Connect additional transformers to feed a three-phase bus feed from another source Relation and phasing Use of instruction, voltmeters, ammeters, phase sequence indicators, etc. Connect current and potential transformers for metering Install and operate voltage regulating apparatus |  | 900 |
| Underground work experience Circuitry of underground equipment System maps and circuit diagrams Underground job layout Subsurface structures and trenches Cable and equipment installation Operations and maintenance of underground system Service drops and meters Safety Substation Orientation Miscellaneous |  |  |
| Totals |  | OJT HRS 4.1K |

LINE ELECTRICIAN

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

49-9051.00

RAPIDS CODE

WSATC-1807

| Related Instruction Online | RSI HRS |
| --- | --- |
| Supervised field trips |  |
| Sponsor approved training seminars Requests for training seminars must be submitted in writing to the committee 30 days prior to the scheduled date of the training seminars. Training Seminars must be preapproved by the committee. |  |
| Sponsor approved online or distance learning courses Requests for online or distance learning courses must be submitted in writing to the committee 30 days prior to the scheduled date of the online or distance learning courses. Online or distance learning courses must be preapproved by the committee. |  |
| State Community/Technical college Seattle Community College |  |
| Other Camp Rilea, Northwest Line Constructors Apprenticeship Electrical Course; and any other training or facilities as approved by the Committee. |  |
| Totals | RSI HRS 0 |